Ganpat University

Faculty of Engineering & Technology

Computer Science & Engineering

Name:- Dwij Vatsal Desai

Sem:-     2

Sub: -   BCS

Enrollment No.:-    23162121027

Prac:-     8

Date:-    22/4/2024

Practical – 8

Aim: To perform Pulse Amplitude Modulation (PAM)/ Sampling theorem  and Demodulation.

Student Group Detail

| No | Enrollment No. | Student Name |
| --- | --- | --- |
| 1 | 23162121027 | Dwij Vatsal Desai |
| 2 | 23162121025 | Kishna Patel |
| 3 | 23162171025 | Virendra rabari |

Apparatus:

1. C.R.O (20MHz)

2. Function generator (1MHz).

3. Connecting cords & probes.

Block Diagram

1. Pulse Amplitude Modulation (PAM)

2. Pulse Amplitude Demodulation

x

Description:

Sampling is a process performed by a sampler. It is present in almost every  type of digital communication system that helps in converting an analog  signal to the digital signal. An analog signal is a continuous time-varying  signal, while digital signal is a signal in the discrete form. The function of a  sampler is to measure the samples of the instantaneous value of the  continuous signal and convert it into the discrete values.

The combination of quantizer and sampler in digital communications  works as an A/D (Analog to Digital) converter. It converts an incoming  analog signal to the digital signal. The sampler converts the analog signal  to the discrete values and the quantizer represent each level to the fixed  discrete finite set of values. Thus, we can define the analog to digital  conversion as a two step process. The first step is performed by the  sampler and the second step is performed by the quantizer.

Nyquist Rate of Sampling

The theoretical minimum sampling rate at which a signal can be sampled  and still can be reconstructed from its samples without any distortion is  called the Nyquist rate of sampling.

Mathematically,

Nyquist Rate fN = 2 ∗ fm

Where fm  is the maximum frequency component present in the signal.

∙ If the signal is sampled at the rate greater than the Nyquist rate, then  the signal is called over sampled.

∙ If the signal is sampled at the rate less than its Nyquist rate, then it  is said to be under sampled.

Expected Waveforms:

Procedure:

∙ Generate audio signal/ Information signal having maximum frequency  fm.

∙ Generate Sampling signal (Pulse train) having frequency more than  the nyquist criteria.

∙ Feed generated signal to PulseAmplitude Modulator (PAM) circuit and  observe the output.

∙ Provide this signal to PAM demodulator circuit and observed the  output.

Observation Table:

| Case | fm | Nyquist Rate fN | fs | Samples in a cycle |
| --- | --- | --- | --- | --- |
| Case 1 | 6 | 6.38 | 1.063 | 24.76 |
| Case 2 | 8 | 10.73 | 1.34 | 37.46 |
| Case 3 | 7 | 18.07 | 2.58 | 50.14 |

Circuit Diagram

Waveform:

Conclusion:

In this practical session, Pulse Amplitude Modulation (PAM) and the Sampling Theorem were studied in depth. Sampling, a crucial process in digital communication, involves converting continuous analog signals into discrete digital form by measuring samples of the signal's instantaneous value. The practical application of this concept was demonstrated through the generation of audio signals, sampling signals (pulse trains), and observation of PAM modulation and demodulation. Additionally, the Nyquist Rate of Sampling was discussed, highlighting its importance in ensuring distortion-free signal reconstruction. Through this practical, students gained hands-on experience and a deeper understanding of these fundamental concepts in digital communication systems.